CHAPTER 2
LITERATURE SURVEY

2.1 Overview of AGV Navigation and Safety Standards

Industrial mobile robot safety is strictly regulated by international safety standards such as ISO 3691-4:2020 (Industrial trucks — Safety requirements and verification — Driverless industrial trucks and their systems). The standard mandates that driverless trucks must be equipped with active personnel detection devices that dynamically adjust protective fields according to speed and steering angle, ensuring the vehicle stops before touching any person or obstacle.

2.2 Sensor Technologies in Mobile Robotics

Various sensing modalities are utilized in industrial mobile robotics:

•	
      Ultrasonic Sensors: Inexpensive and effective for close-range detection, but suffer from low angular resolution, specular reflections, and slow acoustic time-of-flight refresh rates (~20 Hz).

•	
      2D LiDAR and Automotive Radar: Provide high angular resolution and rapid scanning frequencies (20–50 Hz), capable of accurate range and Doppler velocity measurements across wide fields of view (90°–120°).

2.3 Obstacle Avoidance Algorithms

In real-time reactive navigation, the Artificial Potential Field (APF) method, first introduced by O. Khatib (1986), models the robot as a particle moving in a virtual potential field. The destination exerts an attractive force, while obstacles exert repulsive forces inversely proportional to the square of Euclidean distance. While APF is computationally efficient and well-suited for embedded microcontrollers, standard APF can suffer from local minima and force cancellation in narrow corridors (gridlock). Our work extends APF with longitudinal corridor interval slicing to safely handle gridlock scenarios.

2.4 Summary of Gaps in Existing Literature

Many academic prototypes either rely on static geometric safety zones or complex global planners (like A* or RRT) that introduce high computational latency (>100 ms). There is a need for a lightweight, deterministic C++ safety layer that pairs physical stopping distance equations with continuous APF steering and standard POSIX device driver abstractions.