CHAPTER 3
SYSTEM DESIGN & METHODOLOGY

3.1 System Architecture Pipeline

The system is organized into a modular, decoupled pipeline where data flows strictly in one direction:

[ Simulation World ] ──► [ 64-Ray Virtual LiDAR ] ──► [ Linux Character Driver /dev/agv_radar ]

│

▼

[ AGV Kinematics (v,w) ] ◄── [ APF Motion Controller ] ◄── [ Risk Engine & 4-State Safety FSM ]

3.2 Mathematical Formulations

3.2.1 Dynamic Stopping Distance ($d_{stop}$)

The minimum required stopping distance is derived from Newtonian kinematics under constant deceleration:

$$d_{stop}(v) = v ot t_r + rac{v^2}{2a} + d_s$$

Where: $v$ is current velocity (0 to 1.5 m/s), $t_r = 0.2 ext{ s}$ is sensor/controller processing latency, $a = 1.2 ext{ m/s}^2$ is nominal braking deceleration, and $d_s = 0.3 ext{ m}$ is the physical clearance safety margin.

To eliminate threshold hunting during deceleration, thresholds are evaluated at $v_{eval} = max(v_{actual}, v_{nominal})$.

3.2.2 Time-To-Collision (TTC)

For moving obstacles, Time-To-Collision projects relative velocity onto the line-of-sight:

$$ ext{TTC} = rac{ ext{Surface Distance}}{v_{closing}}$$

If $v_{closing} le 0$, the obstacle is stationary or receding, and TTC is assigned $+infty$.

3.2.3 Swept-Volume Corridor Detection

Obstacles are converted into the AGV's local frame $(x_{local}, y_{local})$. To prevent edge-clipping of off-track obstacles, true swept-volume penetration checks:

$$ ext{Lateral Overlap} = |y_{local}| - r_{obstacle} le left( rac{w_{AGV}}{2} + ext{margin} ight)$$

3.2.4 Artificial Potential Field Navigation

The AGV steering is governed by vector summation of forces:

•	
      Goal Attraction: $F_{att, y} = -1.8 ot y_{AGV}$ (restores vehicle to center lane $y = 0$).

•	
      Forward Drive: $F_{att, x} = 3.0$.

•	
      Obstacle Repulsion: $F_{rep} = rac{20.0}{max(0.2, (d - r)^2)}$ pushing laterally away from obstacle centers.