CHAPTER 4
IMPLEMENTATION DETAILS

4.1 Software Development Environment

•	
      Programming Language: C++17 (compiled with Apple Clang / GCC)

•	
      Build Systems: CMake 3.16+ and standard UNIX Makefile

•	
      Platform: Linux / POSIX compliant operating systems

•	
      Simulation Frequency: 20.0 Hz deterministic control loop (50 ms discrete timestep)

4.2 Linux Character Device Driver Emulation (/dev/agv_radar)

To demonstrate embedded systems programming, an emulated character device driver was developed:

•	
      Binary Hardware Packet: Contains magic identifier 0x52414452 ('RADR'), 64-bit microsecond timestamp, 32-bit distance (mm), 16-bit azimuth (mrad), 16-bit speed (mm/s), and an 8-bit XOR checksum.

•	
      Thread Safety: Circular ring buffer protected by std::mutex to prevent race conditions between sensor writes and FSM reads.

•	
      Memory Alignment: Uses std::memcpy for binary serialization, ensuring portability on ARM-based embedded processors (e.g. Raspberry Pi / NVIDIA Jetson).

4.3 Web-Based Interactive Simulator

An interactive HTML5 Canvas simulator (simulator.html) was built using Tailwind CSS and JavaScript:

•	
      64-Ray LiDAR Visualization: Real-time raycasting against circular obstacle geometry.

•	
      Live Risk Heatmap: 4-level color-coded spatial danger overlay.

•	
      Obstacle Path Editor: Interactive right-click waypoint placement for custom patrol routes.

•	
      Telemetry & CSV Export: Real-time oscilloscope graphing and downloadable test telemetry.