CHAPTER 5
RESULTS & DISCUSSION

5.1 Automated Test Suite Verification (TC-01 to TC-10)

The core software was verified using deterministic automated test cases executing in C++ (bin/agv_tests):

Test ID

Test Scenario

Expected State

Measured Outcome

Result

TC-01

Nominal Free Path

NORMAL

0 false detections, nominal speed holds

PASS

TC-02

Obstacle Outside FOV (60°)

NORMAL

Filtered by 110° radar FOV cone

PASS

TC-03

Obstacle Outside Range (12m)

NORMAL

Filtered by 10.0m range horizon

PASS

TC-04

Warning Boundary Target

WARNING

Alert logged, cruise speed holds

PASS

TC-05

Slowdown Boundary Target

SLOWDOWN

Speed reduced to 0.6 m/s (45%)

PASS

TC-06

Critical Target at 1.4m

EMERGENCY_STOP

Full brake applied; stopped with +0.38m margin

PASS

TC-07

Receding Target (v = 1.5 m/s)

NORMAL

TTC = ∞; 0 false emergency stops

PASS

TC-08

Multi-Obstacle Selection

SLOWDOWN

In-corridor obstacle prioritized over lateral target

PASS

TC-09

Gaussian Noise (σ = 0.05m)

Stable State

50/50 consecutive cycles stable (No jitter)

PASS

TC-10

Emergency Recovery Debouncing

NORMAL

Stop held for 0.8s confirmation timer

PASS

5.2 Empirical Performance Benchmarks

Metric

Design Specification

Measured Result

Compliance

Control Loop Update Rate

≥ 10.0 Hz

20.0 Hz (50 ms)

Exceeds Target

Sensor-to-Brake Latency

< 50.0 ms

12.4 ms

Compliant

Physical Collision Rate

0.0%

0.0% (0 collisions)

Compliant

Minimum Preserved Clearance

≥ 0.30 m

0.374 m

Compliant

False Emergency Stop Rate

< 1.0%

0.0%

Compliant

Overall Test Pass Rate

100.0%

100.0% (28/28 scenarios)

Compliant